# Project Category Determination

A structured process is used to determine the Project Category.

- Initial Category Assessment	Assess the roadway and traffic features.

- Risk Analysis	Identify the project-specific risks.

- Final Category Determination	Combine the initial project assessment with the risk

analysis to determine the final project category.

Project Categories are defined as:

  - Category 1 - minimal impact on the travelling public, are typically located on simple terrain, and involve two-lane highways or roads, often with lower speeds and traffic volumes.

  - Category 2 - may be located on higher-speed or higher-volume corridors and involve some complexity. Impacts on the travelling public may be moderate because of the roadway characteristics or the type of work.

  - Category 3 - complex and have a significant impact on the travelling public because of factors such as higher volumes and speeds, project duration, active night work, mountainous terrain, and/or a requirement for lane closures and/or detours.

# Initial Project Category Assessment

The initial project category assessment considers road and traffic characteristics, as well as specific work activities.

Table 3.1: Initial Project Category Assessment on the following pages is used to determine the initial project category.

The total point value calculated at the end of Table 3.1 indicates that the project is initially assessed as a Category 1, 2, or 3.

# Table 3.1: Initial Project Category Assessment

| Traffic Consideration | Value | Point Value | Score |
| --- | --- | --- | --- |
| Posted or Statutory Speed Regular posted speed limit of the roadway | ≤ 50 km/hr | 1 point |  |
|  | 60 - 70 km/hr | 3 points |  |
|  | ≥ 80 km/hr | 4 points |  |
| Traffic Volume Traffic volume (both directions) in peak hours | < 1,000 vehicles/hr | 1 point |  |
|  | 1,000 to 3,000 vehicles/hr | 3 points |  |
|  | > 3,000 vehicles/hr | 4 points |  |
| Lanes Number of lanes in both directions (including auxiliary lanes) | 2 lanes | 0 point |  |
|  | 3 lanes | 2 points |  |
|  | 4 lanes or more | 3 points |  |
| Encroachment Location of work | Off roadway | 0 point |  |
|  | Shoulder work/partial lane closure | 3 points |  |
|  | Full lane closure, ramp closure, or intersection closure | 4 points |  |
| Detours | No detour during construction | 0 point |  |
|  | Detour traffic on temporary roadway during construction next to work zone. | 3 points |  |
|  | Detour route during construction takes traffic off regular route away from work zone; requires detour signing | 4 points |  |
| Duration of Work | Short-duration work (no more than one day-time shift). | 1 point |  |
|  | Long-duration work (less than 2 weeks) | 2 points |  |
|  | Long-duration work (2 or more weeks) | 4 points |  |
| Allowable Delays Delay time plus time to travel through work zone in minutes | < 20 minutes | 1 point |  |
|  | ≥ 20 minutes | 3 points |  |
|  | No allowable delay | 4 points |  |

| Traffic Consideration | Value | Point Value | Score |
| --- | --- | --- | --- |
| Time of Day Time of day that work will occur | Day-time only work | 1 point |  |
|  | Active day-time work, with traffic control devices in place at night | 3 points |  |
|  | Active night-time work | 4 points |  |
| Vertical Alignment | Flat terrain | 0 point |  |
|  | Rolling terrain | 1 point |  |
|  | Mountainous terrain | 2 points |  |
| Horizontal Alignment | Tangent | 0 point |  |
|  | Horizontal curves, no curve advisory speeds | 1 point |  |
|  | Horizontal curves, with curve advisory speeds | 2 points |  |
| Intersections | No intersections or stop- controlled intersection(s) | 0 point |  |
|  | Signalized intersection(s) with no left or right turn phases, or single lane roundabout | 2 points |  |
|  | Signalized intersection(s) with left or right turn phase(s), or multi-lane roundabout | 4 points |  |
|  | Interchange(s) | 5 points |  |
| Runaway Lanes | No runaway lanes | 0 point |  |
|  | Runaway lanes in or near the work zone; they will not be blocked at any time during course of work | 1 point |  |
|  | Runaway lanes in or near work zone; they may be blocked by work or queues during course of work | 4 points |  |
| Pedestrians and Cyclists | No pedestrians or cyclists | 0 point |  |
|  | Possible pedestrians and cyclists | 2 points |  |
|  | Designated cycle route, sidewalk or multi-use pathway | 3 points |  |

| Traffic Consideration | Value | Point Value | Score |
| --- | --- | --- | --- |
| HOV or Bus Lane | No HOV or bus lane | 0 point |  |
|  | HOV or bus lane | 4 points |  |
| Counter-Flow Lane | No counter-flow lane | 0 point |  |
|  | Counter-flow lane | 4 points |  |
|  |  | Total Score |  |
|  |  | Category 1 | < 16 |
|  |  | Category 2 | 16 to 25 |
|  |  | Category 3 | > 25 |
|  |  | Initial Project Category |  |

    - Project Risk Analysis

A project risk analysis is the process of reviewing site-specific characteristics and considering the likelihood and consequence of each item listed. It is able to highlight potential hazards that are not captured in the Initial Project Category Assessment.

Each project has a unique combination of site-specific characteristics, and the risk analysis considers potential hazards associated with the specific project and/or location.

Table 3.2: Project Risk Analysis on the following pages is used to determine whether each potential hazard creates a low, medium, or high risk for the project and location.

The total point value calculated at the end of Table 3.2 indicates that the project is assessed as a low-risk, medium-risk, or high-risk project.

Combining the results of the initial project category assessment and the risk analysis will determine the final project category (see Section 3.3.3: Final Project Category Determination).

# Table 3.2: Project Risk Analysis

The Project Risk Analysis is a general guideline, applicable to most projects. If significant project-specific hazards are not included in the risk analysis below, the Evaluator may consider increasing the final risk rating. This modification and the justification for it should be documented.

All high-risk, project-specific hazards should be addressed and mitigated in the Traffic Management Plan.

| Item | Risk | Definition | Point Value | Score |
| --- | --- | --- | --- | --- |
| Falling object | Low | Potential of falling object through course of work (i.e., overhead works, slung loads, or equipment boom/bucket work) | 1 point |  |
|  | Medium | Working within a known avalanche or rock fall area; no recent evidence of activity | 2 points |  |
|  | High | Recent evidence of rock or material entering work site or overhead work that may impact travelling public or worker safety (i.e., overhead structures) Vehicle queues may back into a rock fall or avalanche area | 3 points |  |
| Nature of work activity | Low | Work activity is not expected to create a significant hazard | 1 point |  |
|  | Medium | Work activity will create excessive dirt, dust, or gravel on the road surface, and will thereby create a potential hazard | 2 points |  |
|  | High | Work activity such as blasting, scaling, or excavation < 2 metres from active travelling lanes will create a potential hazard | 3 points |  |
| Removal of safety devices | Low | No removal of safety devices | 1 point |  |
|  | Medium | Removal of safety devices such as pavement markings, signage, traffic signal, or reflectors | 2 points |  |
|  | High | Removal of containment devices, such as barrier, guard rail, crash attenuators, fencing, etc. | 3 points |  |
| Equipment movement through work zone | Low | Minimal conflict with traffic (e.g., work commencing off travelled roadway) | 1 point |  |
|  | Medium | Conflict with normal traffic flow; no queuing or traffic stoppages | 2 points |  |
|  | High | Conflicts with normal traffic; may create queuing and require traffic stoppages. Difficult for equipment to enter and exit site | 3 points |  |

| Item | Risk | Definition | Point Value | Score |
| --- | --- | --- | --- | --- |
| Roadway surface condition during construction | Low | Roadway surface is maintained | 1 point |  |
|  | Medium | Roadway surface, such as milling and grinding (consistent surface), creates a hazard for road users | 2 points |  |
|  | High | Roadway surface is inconsistent, with multiple changes or work tasks (manholes, culvert installation, etc.) | 3 points |  |
| Storage of equipment and material | Low | Stored outside the shoulder | 1 point |  |
|  | Medium | Stored on the shoulder but outside travelled roadway | 2 points |  |
|  | High | Stored on shoulder but encroaching on travelled roadway | 3 points |  |
| Load restrictions as a result of construction | Low | No load restrictions | 1 point |  |
|  | Medium | Narrow lanes restrict wide loads | 2 points |  |
|  | High | Overweight/overheight vehicles restricted (may result in structural damage) | 3 points |  |
| Lane widths | Low | Maintain existing lane widths | 1 point |  |
|  | Medium | n/a | n/a |  |
|  | High | Lane width not maintained throughout work zone, or Single-lane alternating traffic | 3 points |  |
| Work zone or queues block access (active or inactive site) | Low | None | 1 point |  |
|  | Medium | Side street or business access | 2 points |  |
|  | High | Major public facility and/or major secondary roadway | 3 points |  |
| Transit access | Low | No transit or school bus stops | 1 point |  |
|  | Medium | Community shuttle or school bus stops | 2 points |  |
|  | High | Express transit or major bus route | 3 points |  |
| Impacts of special events | Low | No known event | 1 point |  |
|  | Medium | Moderate public event with attendance under 5,000 | 2 points |  |
|  | High | Major public event with attendance over 5,000 or moderate public event (under 5,000) with no alternative access or route | 3 points |  |

| Item | Risk | Definition | Point Value | Score |
| --- | --- | --- | --- | --- |
| Overlapping work | Low | No overlapping work | 1 point |  |
|  | Medium | Another work site within 3 km; traffic control for the projects could impact one another | 2 points |  |
|  | High | Work sites adjacent or overlapping | 3 points |  |
| Emergency facility (ie. hospital, police, ambulance, and fire stations) | Low | No emergency facility near work site | 1 point |  |
|  | Medium | 24-hour manned emergency facility | 2 points |  |
|  | High | Volunteer-staffed emergency facility; consider responder access through work zone to the facility, and emergency response from facility through the work zone | 3 points |  |
|  |  |  | Total Score |  |
|  |  |  | Low Risk | < 23 |
|  |  |  | Medium Risk | 23 to 28 |
|  |  |  | High Risk | > 28 |
|  |  |  | Project Risk |  |

# Final Project Category Determination

The matrix in Table 3.3: Final Project Category Determination should be used to make the final project category determination.

It combines the initial project category assessment with the results of the risk analysis

to identify a final project category based on roadway and traffic characteristics and risks.

It may be appropriate to increase the final category level for high-risk projects to reflect the complexity or hazards associated with the work.

# Table 3.3: Final Project Category Determination

|  |  | Initial Project Category Assessment |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 |
| Project Risk | Low | Category 1 | Category 2 | Category 3 |
|  | Medium | Category 1 | Category 2 | Category 3 |
|  | High | Category 2 | Category 3 | Category 3 |

The final project category determination should be used to identify required and recommended sub-plans and special conditions addressed in the Traffic Management Plan.

This process is a guide and may not capture all components of the project which should be considered when determining the Project Category.